ATLAS GUARD TOWER (AGT) — INTERVIEW MASTER GUIDE

Defense-Tech Edge AI Perimeter Surveillance & LoRa Mesh Sensor Node

1. CORE TECHNICAL IMPLEMENTATION QUESTIONS

Q1: How did you physically attach and code the web camera on the Raspberry Pi?

Answer & Architecture:

• Hardware Interface: Connected via high-speed USB 3.0 port (or MIPI-CSI 15-pin ribbon cable). The Linux kernel enumerates the camera under the Video4Linux2 (V4L2) subsystem at /dev/video0.

• Frame Buffer Lag Elimination: Standard cv2.VideoCapture on Linux queues 4-5 frames in an OS buffer, causing multi-second delay on embedded devices. We resolved this by implementing a dedicated ThreadedCameraWorker in Python that runs in the background, sets cv2.CAP_PROP_BUFFERSIZE=1, and continuously drains the buffer. When an interrupt arrives, the AI pipeline immediately gets the latest instantaneous frame.

• Python Code Used: Uses cv2.VideoCapture(0, cv2.CAP_V4L2) with a thread-safe get_latest_frame() lock mechanism.

Q2: Since YOLO detects 80 COCO classes, how did you filter exclusively for humans?

Answer & Logic:

• Target Class Gating: Filtered results strictly for COCO Class ID 0 (which corresponds to 'person'). All other 79 classes (cars, animals, chairs, etc.) are ignored.

• Confidence & Gating: Implemented a confidence threshold (>= 0.55) and multi-frame temporal confirmation. When the microwave radar triggers, the node evaluates 5 consecutive frames. If at least 2 frames confirm human presence, the threat is verified. Transient noise, birds, or blowing leaves are filtered out as false alarms.

Q3: How did you integrate Raspberry Pi data with Heltec LoRa32 running stock Meshtastic firmware?

Answer & Networking:

• Interconnect: Heltec LoRa32 V3 is connected to the Pi via USB-C to USB-A (/dev/ttyUSB0).

• Communication Protocol: Stock Meshtastic firmware exposes a standardized Protocol Buffers (Protobuf) RPC API over USB-serial. No firmware modification is needed.

• Python SDK: We use the official 'meshtastic' Python library (meshtastic.serial_interface.SerialInterface). The Pi calls interface.sendText(payload, destinationId='^all', channelIndex=0, wantAck=True).

• What the Heltec Handles: The ESP32-S3 and SX1262 transceiver handle AES-256 encryption, LoRa framing, RF modulation over 868 MHz / IN_865 band, and multi-hop mesh routing.

Q4: Which GPS sensor did you use and which library on Raspberry Pi parsed the data?

Answer & Serial Setup:

• Hardware: u-blox NEO-6M GPS receiver connected to Hardware UART pins (GPIO 14 TX -> GPS RX, GPIO 15 RX -> GPS TX).

• Linux OS Configuration: Enabled UART in /boot/firmware/config.txt (enable_uart=1) and removed the Linux serial login console from /boot/firmware/cmdline.txt (removed console=serial0,115200) to dedicate /dev/serial0 exclusively to GPS.

• Python Libraries: Used 'pyserial' at 9600 baud and 'pynmea2' to parse $GPGGA (coordinates, altitude, satellites) and $GPRMC (validity flag 'A' vs 'V', UTC timestamp).

2. LINUX OS STARTUP & FIELD RELIABILITY

Q5: How do you make the program auto-start when the Raspberry Pi powers on in the field?

Answer: Created a systemd unit service (/etc/systemd/system/agt-sentry.service) with 'Restart=always' and 'WantedBy=multi-user.target', enabled via 'sudo systemctl enable agt-sentry.service'.

Q6: How do you prevent SD card corruption if solar battery power dies abruptly?

Answer: Enabled Linux Read-Only Root Filesystem (OverlayFS) using raspi-config. Temporary buffers and logs are written to RAM (tmpfs).

3. SENSOR FUSION & C2 PLATFORM (HIVEOS)

Q7: What is the 4-stage gated pipeline architecture?

Answer: Stage 1 (Passive Radar: RCWL-0516 Doppler interrupt) -> Stage 2 (Active AI: YOLOv8n 5-frame human check) -> Stage 3 (Geotagging: NEO-6M GPS) -> Stage 4 (LoRa Mesh Dispatch: HiveOS CDM format via Heltec LoRa32).

Q8: What is the Common Data Model (CDM) and HiveOS?

Answer: Modeled conceptually on defense systems like Anduril's Lattice, the Common Data Model is a unified data schema (ThreatEvent) for tracks and alerts. HiveOS ingests these tracks for operator visualization. Crucially, a Human-in-the-Loop authorizes any response.